ISCW LAB: MPLS VPN ROUTING VRF

(dùng EIGRP)

---o0o---

- Mô hình:

Qui hoạch IP:

- 192.168.1.0/24

- 192.168.2.0/24

- 192.168.3.0/24

- 192.168.4.0/24

- 192.168.5.0/24

- 192.168.6.0/24

- Lo0: 10.10.10.10/24

- Lo0: 20.20.20.20/24

- Lo0: 1.1.1.1/24

- Lo0: 3.3.3.3/24

- Lo0: 2.2.2.2/24

- Lo0: 30.30.30.30/24

- Lo0: 40.40.40.40/24

- Yêu cầu:

  - Cấu hình MPLS domain giữa PE01 – P – PE02 (IGP = RIPv2)

  - Cấu hình BGP AS 1 giữa PE01 và PE 02

  - Trên PE01 tạo vrf A1 và B1 tương ứng với mỗi Router A1,B1

  - Trên PE02 tạo vrf A2 và B2 tương ứng với mỗi Router A2,B2

  - A1 – A2 – B1 – B2 chạy giao thức định tuyến EIGRP

    - A1 : AS = 10

    - A2 : AS = 30

    - B1 : AS = 20

    - B2 : AS = 40

- Cấu hình:

ROUTER A1:

!

hostname A1

!

boot-start-marker

boot-end-marker

!

!

no aaa new-model

ip subnet-zero

ip cef

!

!

!

no ip domain lookup

ip audit po max-events 100

!

!

interface Loopback0

ip address 10.10.10.10 255.255.255.0

!

interface FastEthernet0/0

ip address 192.168.1.1 255.255.255.0

duplex auto

speed auto

!

interface FastEthernet0/1

no ip address

shutdown

duplex auto

speed auto

!

router eigrp 10

network 10.0.0.0

network 192.168.1.0

no auto-summary

!

ip classless

!

ip http server

no ip http secure-server

!

!

line con 0

exec-timeout 0 0

line aux 0

line vty 0 4

privilege level 15

no login

line vty 5 15

privilege level 15

no login

!

End

ROUTER B1:

hostname B1

!

boot-start-marker

boot-end-marker

!

!

no aaa new-model

ip subnet-zero

ip cef

!

!

!

no ip domain lookup

ip audit po max-events 100

!

!

interface Loopback0

ip address 20.20.20.20 255.255.255.0

!

interface FastEthernet0/0

no ip address

shutdown

duplex auto

speed auto

!

interface Serial0/0

ip address 192.168.2.1 255.255.255.0

clock rate 2000000

!

interface FastEthernet0/1

ip address 100.100.100.100 255.255.255.0

duplex auto

speed auto

no keepalive

!

interface Serial0/1

no ip address

shutdown

clock rate 2000000

!

router eigrp 20

network 20.0.0.0

network 100.0.0.0

network 192.168.2.0

no auto-summary

!

ip classless

!

ip http server

no ip http secure-server

!

!

line con 0

exec-timeout 0 0

line aux 0

line vty 0 4

privilege level 15

no login

line vty 5 15

privilege level 15

no login

!

End

ROUTER PE01:

hostname PE01

!

boot-start-marker

boot-end-marker

!

!

no aaa new-model

ip subnet-zero

ip cef

!

!

!

no ip domain lookup

ip vrf A1

rd 1:100

route-target export 1:100

route-target import 1:100

!

ip vrf B1

rd 1:200

route-target export 1:200

route-target import 1:200

!

ip audit po max-events 100

!

!

interface Loopback0

ip address 1.1.1.1 255.255.255.0

!

interface FastEthernet0/0

ip vrf forwarding A1

ip address 192.168.1.2 255.255.255.0

duplex auto

speed auto

!

interface Serial0/0

ip vrf forwarding B1

ip address 192.168.2.2 255.255.255.0

clock rate 64000

!

interface FastEthernet0/1

ip address 192.168.3.1 255.255.255.0

duplex auto

speed auto

mpls label protocol ldp

tag-switching ip

!

interface Serial0/1

no ip address

shutdown

clock rate 2000000

!

router eigrp 100

auto-summary

!

address-family ipv4 vrf B1

network 192.168.2.0

no auto-summary

autonomous-system 20

exit-address-family

!

address-family ipv4 vrf A1

network 192.168.1.0

no auto-summary

autonomous-system 10

exit-address-family

!

router rip

version 2

network 1.0.0.0

network 192.168.3.0

no auto-summary

!

router bgp 1

no synchronization

bgp log-neighbor-changes

neighbor 2.2.2.2 remote-as 1

neighbor 2.2.2.2 update-source Loopback0

no auto-summary

!

address-family vpnv4

neighbor 2.2.2.2 activate

neighbor 2.2.2.2 send-community extended

exit-address-family

!

address-family ipv4 vrf B1

redistribute eigrp 20

no auto-summary

no synchronization

exit-address-family

!

address-family ipv4 vrf A1

redistribute eigrp 10

no auto-summary

no synchronization

exit-address-family

!

ip classless

!

ip http server

no ip http secure-server

!

!

line con 0

exec-timeout 0 0

line aux 0

line vty 0 4

privilege level 15

no login

line vty 5 15

privilege level 15

no login

!

End

ROUTER P:

hostname P

!

boot-start-marker

boot-end-marker

!

!

no aaa new-model

ip subnet-zero

ip cef

!

!

!

no ip domain lookup

ip audit po max-events 100

!

interface Loopback0

ip address 3.3.3.3 255.255.255.0

!

interface FastEthernet0/0

ip address 192.168.3.2 255.255.255.0

duplex auto

speed auto

mpls label protocol ldp

tag-switching ip

!

interface FastEthernet0/1

ip address 192.168.4.1 255.255.255.0

duplex auto

speed auto

mpls label protocol ldp

tag-switching ip

!

router rip

version 2

network 3.0.0.0

network 192.168.3.0

network 192.168.4.0

no auto-summary

!

ip classless

!

ip http server

no ip http secure-server

!

!

line con 0

exec-timeout 0 0

line aux 0

line vty 0 4

privilege level 15

no login

line vty 5 15

privilege level 15

no login

!

End

ROUTER PE02:

!

hostname PE02

!

boot-start-marker

boot-end-marker

!

!

no aaa new-model

ip subnet-zero

ip cef

!

!

!

no ip domain lookup

ip vrf A2

rd 1:100

route-target export 1:100

route-target import 1:100

!

ip vrf B2

rd 1:200

route-target export 1:200

route-target import 1:200

!

ip audit po max-events 100

!

!

interface Loopback0

ip address 2.2.2.2 255.255.255.0

!

interface FastEthernet0/0

ip address 192.168.4.2 255.255.255.0

duplex auto

speed auto

mpls label protocol ldp

tag-switching ip

!

interface Serial0/0

ip vrf forwarding A2

ip address 192.168.5.1 255.255.255.0

clock rate 64000

!

interface FastEthernet0/1

ip vrf forwarding B2

ip address 192.168.6.1 255.255.255.0

duplex auto

speed auto

!

interface Serial0/1

no ip address

shutdown

clock rate 2000000

!

router eigrp 100

auto-summary

!

address-family ipv4 vrf B2

redistribute bgp 1 metric 1000 100 100 100 100

network 192.168.6.0

auto-summary

autonomous-system 40

exit-address-family

!

address-family ipv4 vrf A2

redistribute bgp 1 metric 1000 100 100 100 100

network 192.168.5.0

auto-summary

autonomous-system 30

exit-address-family

!

router rip

version 2

network 2.0.0.0

network 192.168.4.0

no auto-summary

!

router bgp 1

no synchronization

bgp log-neighbor-changes

neighbor 1.1.1.1 remote-as 1

neighbor 1.1.1.1 update-source Loopback0

no auto-summary

!

address-family vpnv4

neighbor 1.1.1.1 activate

neighbor 1.1.1.1 send-community extended

exit-address-family

!

address-family ipv4 vrf B2

redistribute eigrp 40

no auto-summary

no synchronization

exit-address-family

!

address-family ipv4 vrf A2

redistribute eigrp 30

no auto-summary

no synchronization

exit-address-family

!

ip classless

!

ip http server

no ip http secure-server

!

!

line con 0

exec-timeout 0 0

line aux 0

line vty 0 4

privilege level 15

no login

line vty 5 15

privilege level 15

no login

!

End

ROUTER A2:

!

hostname A2

!

boot-start-marker

boot-end-marker

!

!

no aaa new-model

ip subnet-zero

ip cef

!

!

!

no ip domain lookup

ip audit po max-events 100

!

interface Loopback0

ip address 30.30.30.30 255.255.255.0

!

interface FastEthernet0/0

no ip address

shutdown

duplex auto

speed auto

!

interface Serial0/0

ip address 192.168.5.2 255.255.255.0

clock rate 2000000

!

interface FastEthernet0/1

no ip address

shutdown

duplex auto

speed auto

!

interface Serial0/1

no ip address

shutdown

clock rate 2000000

!

router eigrp 30

network 30.0.0.0

network 192.168.5.0

no auto-summary

!

ip classless

!

ip http server

no ip http secure-server

!

!

line con 0

exec-timeout 0 0

line aux 0

line vty 0 4

privilege level 15

no login

line vty 5 15

privilege level 15

no login

!

End

ROUTER B2:

!

hostname B2

!

boot-start-marker

boot-end-marker

!

!

no aaa new-model

ip subnet-zero

ip cef

!

!

!

no ip domain lookup

ip audit po max-events 100

!

!

interface Loopback0

ip address 40.40.40.40 255.255.255.0

!

interface FastEthernet0/0

ip address 192.168.6.2 255.255.255.0

duplex auto

speed auto

!

interface FastEthernet0/1

no ip address

shutdown

duplex auto

speed auto

!

router eigrp 40

network 40.0.0.0

network 192.168.6.0

no auto-summary

!

ip classless

!

ip http server

no ip http secure-server

!

!

line con 0

exec-timeout 0 0

line aux 0

line vty 0 4

privilege level 15

no login

line vty 5 15

privilege level 15

no login

!

End

- Kiểm tra hoạt động:

  - Đối với MPLS Domain :

    - Kiểm tra bảng định tuyến :

Trên PE01

PE01#show ip route

Gateway of last resort is not set

1.0.0.0/24 is subnetted, 1 subnets

C       1.1.1.0 is directly connected, Loopback0

2.0.0.0/24 is subnetted, 1 subnets

R       2.2.2.0 [120/2] via 192.168.3.2, 00:00:12, FastEthernet0/1

3.0.0.0/24 is subnetted, 1 subnets

R       3.3.3.0 [120/1] via 192.168.3.2, 00:00:12, FastEthernet0/1

R    192.168.4.0/24 [120/1] via 192.168.3.2, 00:00:12, FastEthernet0/1

C    192.168.3.0/24 is directly connected, FastEthernet0/1

Trên P

P#show ip route

Gateway of last resort is not set

1.0.0.0/24 is subnetted, 1 subnets

R       1.1.1.0 [120/1] via 192.168.3.1, 00:00:07, FastEthernet0/0

2.0.0.0/24 is subnetted, 1 subnets

R       2.2.2.0 [120/1] via 192.168.4.2, 00:00:11, FastEthernet0/1

3.0.0.0/24 is subnetted, 1 subnets

C       3.3.3.0 is directly connected, Loopback0

C    192.168.4.0/24 is directly connected, FastEthernet0/1

C    192.168.3.0/24 is directly connected, FastEthernet0/0

Trên PE02

PE02#show ip route

Gateway of last resort is not set

1.0.0.0/24 is subnetted, 1 subnets

R       1.1.1.0 [120/2] via 192.168.4.1, 00:00:27, FastEthernet0/0

2.0.0.0/24 is subnetted, 1 subnets

C       2.2.2.0 is directly connected, Loopback0

3.0.0.0/24 is subnetted, 1 subnets

R       3.3.3.0 [120/1] via 192.168.4.1, 00:00:27, FastEthernet0/0

C    192.168.4.0/24 is directly connected, FastEthernet0/0

R    192.168.3.0/24 [120/1] via 192.168.4.1, 00:00:27, FastEthernet0/0

Kết luận : Định tuyến nội giữa MPLS Domain đã thông, từ đó cấu hình MPLS

    - Kiểm tra LFIP table

Trên PE01

PE01#show mpls forwarding-table

Local  Outgoing    Prefix            Bytes tag  Outgoing   Next Hop

tag    tag or VC   or Tunnel Id      switched   interface

16     16          2.2.2.0/24        0          Fa0/1      192.168.3.2

17     Pop tag     3.3.3.0/24        0          Fa0/1      192.168.3.2

18     Pop tag     192.168.4.0/24    0          Fa0/1      192.168.3.2

Trên P

P#show mpls forwarding-table

Local  Outgoing    Prefix            Bytes tag  Outgoing   Next Hop

tag    tag or VC   or Tunnel Id      switched   interface

16     Pop tag     2.2.2.0/24        3375       Fa0/1      192.168.4.2

17     Pop tag     1.1.1.0/24        5007       Fa0/0      192.168.3.1

Trên PE02

PE02#show mpls forwarding-table

Local  Outgoing    Prefix            Bytes tag  Outgoing   Next Hop

tag    tag or VC   or Tunnel Id      switched   interface

16     Pop tag     3.3.3.0/24        0          Fa0/0      192.168.4.1

17     Pop tag     192.168.3.0/24    0          Fa0/0      192.168.4.1

18     17          1.1.1.0/24        0          Fa0/0      192.168.4.1

    - Cấu hình BGP trên PE01 và PE02

Trên PE01

PE01#show ip bgp summary

BGP router identifier 1.1.1.1, local AS number 1

BGP table version is 1, main routing table version 1

Neighbor        V    AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

2.2.2.2         4     1      55      54        1    0    0 00:43:36        0

Trên PE02

PE02#show ip bgp summary

BGP router identifier 2.2.2.2, local AS number 1

BGP table version is 1, main routing table version 1

Neighbor        V    AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

1.1.1.1         4     1      55      56        1    0    0 00:44:24        0

- Kiểm tra vrf routing table của A1,A2,B1,B2 trên PE01 và PE02

Trên PE01:

PE01#show ip route vrf A1

Routing Table: A1

Gateway of last resort is not set

B    192.168.5.0/24 [200/0] via 2.2.2.2, 00:45:07

10.0.0.0/24 is subnetted, 1 subnets

D       10.10.10.0 [90/156160] via 192.168.1.1, 00:45:24, FastEthernet0/0

C    192.168.1.0/24 is directly connected, FastEthernet0/0

30.0.0.0/24 is subnetted, 1 subnets

B       30.30.30.0 [200/2297856] via 2.2.2.2, 00:45:07

PE01#show ip route vrf B1

Routing Table: B1

Gateway of last resort is not set

100.0.0.0/24 is subnetted, 1 subnets

D       100.100.100.0 [90/2172416] via 192.168.2.1, 00:44:46, Serial0/0

20.0.0.0/24 is subnetted, 1 subnets

D       20.20.20.0 [90/2297856] via 192.168.2.1, 00:44:46, Serial0/0

40.0.0.0/24 is subnetted, 1 subnets

B       40.40.40.0 [200/156160] via 2.2.2.2, 00:46:20

B    192.168.6.0/24 [200/0] via 2.2.2.2, 00:46:20

C    192.168.2.0/24 is directly connected, Serial0/0

Trên PE02:

PE02#show ip route vrf A2

Routing Table: A2

Gateway of last resort is not set

C    192.168.5.0/24 is directly connected, Serial0/0

10.0.0.0/24 is subnetted, 1 subnets

B       10.10.10.0 [200/156160] via 1.1.1.1, 00:45:16

B    192.168.1.0/24 [200/0] via 1.1.1.1, 00:45:16

30.0.0.0/24 is subnetted, 1 subnets

D       30.30.30.0 [90/2297856] via 192.168.5.2, 00:48:47, Serial0/0

PE02#show ip route vrf B2

Routing Table: B2

Codes: C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route

Gateway of last resort is not set

100.0.0.0/24 is subnetted, 1 subnets

B       100.100.100.0 [200/2172416] via 1.1.1.1, 00:46:44

20.0.0.0/24 is subnetted, 1 subnets

B       20.20.20.0 [200/2297856] via 1.1.1.1, 00:46:44

40.0.0.0/24 is subnetted, 1 subnets

D       40.40.40.0 [90/156160] via 192.168.6.2, 00:49:30, FastEthernet0/1

C    192.168.6.0/24 is directly connected, FastEthernet0/1

B    192.168.2.0/24 [200/0] via 1.1.1.1, 00:45:59

- Kiểm tra thông tuyến:

A1#ping 30.30.30.30

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 30.30.30.30, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 196/236/300 ms

B1#ping 40.40.40.40

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 40.40.40.40, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 156/180/200 ms

- Kiểm tra bảng LFIP hoàn chỉnh trên mỗi Router PE01 và PE02:

PE01#show mpls forwarding-table

Local  Outgoing    Prefix            Bytes tag  Outgoing   Next Hop

tag    tag or VC   or Tunnel Id      switched   interface

16     16          2.2.2.0/24        0          Fa0/1      192.168.3.2

17     Pop tag     3.3.3.0/24        0          Fa0/1      192.168.3.2

18     Pop tag     192.168.4.0/24    0          Fa0/1      192.168.3.2

19     Untagged    20.20.20.0/24[V]  0          Se0/0      point2point

20     Untagged    100.100.100.0/24[V]   \

0          Se0/0      point2point

21     Untagged    10.10.10.0/24[V]  0          Fa0/0      192.168.1.1

22     Aggregate   192.168.1.0/24[V] 520

23     Aggregate   192.168.2.0/24[V] 520

PE02#show mpls forwarding-table

Local  Outgoing    Prefix            Bytes tag  Outgoing   Next Hop

tag    tag or VC   or Tunnel Id      switched   interface

16     Pop tag     3.3.3.0/24        0          Fa0/0      192.168.4.1

17     Pop tag     192.168.3.0/24    0          Fa0/0      192.168.4.1

18     17          1.1.1.0/24        0          Fa0/0      192.168.4.1

19     Untagged    30.30.30.0/24[V]  520        Se0/0      point2point

20     Aggregate   192.168.5.0/24[V] 0

21     Untagged    40.40.40.0/24[V]  570        Fa0/1      192.168.6.2

22     Aggregate   192.168.6.0/24[V] 0

- Kiểm tra cơ chế chuyển mạch nhãn tại MPLS Domain

PE01#traceroute vrf A1 30.30.30.30

Type escape sequence to abort.

Tracing the route to 30.30.30.30

1 192.168.3.2 [MPLS: Labels 16/19 Exp 0] 160 msec 184 msec 144 msec

2 192.168.5.1 [MPLS: Label 19 Exp 0] 128 msec 60 msec 92 msec

3 192.168.5.2 176 msec *  216 msec

PE02#traceroute vrf B2 20.20.20.20

Type escape sequence to abort.

Tracing the route to 20.20.20.20

1 192.168.4.1 [MPLS: Labels 17/19 Exp 0] 176 msec 120 msec 156 msec

2 192.168.2.2 [MPLS: Label 19 Exp 0] 212 msec 64 msec 52 msec

3 192.168.2.1 92 msec *  160 msec